# Capability Requirements

## IMU Module Capability Requirements

Interface module for control data between the IMU and the P3 Weapon.

The P3 MCP IMU Module shall provide the following capabilities:

IMU Initialization State

Interrupt Handler For Imu Communication

IMU Communication Status Check

## IMU Initialization State

1. The IMU module shall initialize the SDLC interface using the FPGA address 0x41800000 and assign the IMU interrupt handler to manage communication interrupts.

## Interrupt Handler For Imu Communication

2. The IMU module shall validate the received message address against the expected IMU address 0x0A.

3. The IMU module shall process a message with ID 0x01 if the message length matches the size of the first message type.

4. The IMU module shall increment the error counter when a sequence number error is detected for a message with ID 0x01.

5. The IMU module shall update the raw data with the contents of the first message type.

6. The IMU module shall compute the angular rate for the X-axis using a scaling factor of 0.00457763671875.

7. The IMU module shall compute the angular rate for the Z-axis using a scaling factor of 0.0011444091796875.

8. The IMU module shall compute the angular rate for the Y-axis using a scaling factor of 0.0011444091796875.

9. The IMU module shall compute the acceleration for the X-axis using a scaling factor of 0.02232421875.

10. The IMU module shall compute the acceleration for the Z-axis using a scaling factor of 0.02232421875.

11. The IMU module shall compute the acceleration for the Y-axis using a scaling factor of 0.02232421875.

12. The IMU module shall mark the data as valid after processing a message with ID 0x01.

13. The IMU module shall increment the cycle counter after processing a message with ID 0x01.

14. The IMU module shall reset the cycle counter to zero when it reaches a value of 6 after processing a message with ID 0x01.

15. The IMU module shall record the timestamp of the received message after processing a message with ID 0x01.

16. The IMU module shall output diagnostic information to the auxiliary system when a message with ID 0x01 is received and the receive auxiliary function is enabled.

17. The IMU module shall output detailed diagnostic information to the auxiliary system when a message with ID 0x01 is received and the detailed receive auxiliary function is enabled.

18. The IMU module shall process a message with ID 0x02 if the message length matches the size of the second message type.

19. The IMU module shall increment the error counter when a sequence number error is detected for a message with ID 0x02.

20. The IMU module shall update the raw data with the contents of the second message type.

21. The IMU module shall compute the angular rate for the X-axis using a scaling factor of 0.00457763671875 for a message with ID 0x02.

22. The IMU module shall compute the angular rate for the Z-axis using a scaling factor of 0.0011444091796875 for a message with ID 0x02.

23. The IMU module shall compute the angular rate for the Y-axis using a scaling factor of 0.0011444091796875 for a message with ID 0x02.

24. The IMU module shall compute the acceleration for the X-axis using a scaling factor of 0.02232421875 for a message with ID 0x02.

25. The IMU module shall compute the acceleration for the Z-axis using a scaling factor of 0.02232421875 for a message with ID 0x02.

26. The IMU module shall compute the acceleration for the Y-axis using a scaling factor of 0.02232421875 for a message with ID 0x02.

27. The IMU module shall compute the delta angle for the X-axis using a scaling factor of (1.0 / 1073741824) for a message with ID 0x02.

28. The IMU module shall compute the delta angle for the Z-axis using a scaling factor of (1.0 / 4294967296) for a message with ID 0x02.

29. The IMU module shall compute the delta angle for the Y-axis using a scaling factor of (1.0 / 4294967296) for a message with ID 0x02.

30. The IMU module shall compute the delta velocity for the X-axis using a scaling factor of (0.3048 / 67108864) for a message with ID 0x02.

31. The IMU module shall compute the delta velocity for the Z-axis using a scaling factor of (0.3048 / 67108864) for a message with ID 0x02.

32. The IMU module shall compute the delta velocity for the Y-axis using a scaling factor of (0.3048 / 67108864) for a message with ID 0x02.

33. The IMU module shall mark the data as valid after processing a message with ID 0x02.

34. The IMU module shall reset the cycle counter to zero after processing a message with ID 0x02.

35. The IMU module shall record the timestamp of the received message after processing a message with ID 0x02.

36. The IMU module shall output diagnostic data in CSV format to the auxiliary system when a message with ID 0x02 is received and the auxiliary to CSV function is enabled.

37. The IMU module shall output diagnostic information to the auxiliary system when a message with ID 0x02 is received and the receive auxiliary function is enabled.

38. The IMU module shall output detailed diagnostic information to the auxiliary system when a message with ID 0x02 is received and the detailed receive auxiliary function is enabled.

39. The IMU module shall increment the error counter when an invalid message ID is received.

40. The IMU module shall output diagnostic information to the auxiliary system when an invalid message ID is received and error detection is enabled.

41. The IMU module shall increment the error counter when an invalid IMU address is received.

42. The IMU module shall output diagnostic information to the auxiliary system when an invalid IMU address is received and error detection is enabled.

43. The IMU module shall increment the error counter when a CRC error is detected.

44. The IMU module shall output diagnostic information to the auxiliary system when a CRC error is detected and error detection is enabled.

45. The IMU module shall increment the total received message counter after processing each interrupt.

46. The IMU module shall schedule the 600Hz task after processing each interrupt.

## IMU Communication Status Check

1. The IMU module shall disable interrupts before performing communication checks.

2. The IMU module shall determine that 2100 microseconds have elapsed since the last timestamp update.

3. The IMU module shall update the timestamp to the current system time.

4. The IMU module shall mark the data as invalid.

5. The IMU module shall increment the error counter by 1.

6. The IMU module shall increment the cycle counter by 1.

7. The IMU module shall output an error message indicating that IMU data is invalid if the error detection is enabled.

8. The IMU module shall reset the cycle counter to 0 if the cycle counter reaches 6.

9. The IMU module shall schedule the 600Hz task for execution.

10. The IMU module shall re-enable interrupts after performing communication checks.