# Capability Requirements

## Main Module Capability Requirements

Main function of P3

The P3 MCP Main Module shall provide the following capabilities:

System Wide Initialization State

ARCNET Node Status Check

Application Execution Management

## System Wide Initialization State

1. The system shall set the auxiliary power control to enable.

2. The system shall disable the node check control.

3. The system shall disable the X configuration control.

4. The system shall disable the database access control.

5. The system shall enable the WIAC82 control.

6. The system shall disable the P3 light control.

7. The system shall disable the shallow control.

8. The system shall enable the MPSAL control.

9. The system shall disable the QSAL control.

10. The system shall disable the DSAL control.

11. The system shall enable the IMU20 control.

12. The system shall disable the IMU15 control.

13. The system shall enable the GNSS control.

14. The system shall indicate that the fuze card is fitted.

15. The system shall set the system time to the current time in microseconds.

## ARCNET Node Status Check

1. The system shall evaluate ARCNET node IDs ranging from 240 to 255 inclusive.

2. The system shall determine whether each ARCNET node ID is present on the network.

3. For each ARCNET node ID that is present on the network, the system shall generate a message indicating the node ID and its hexadecimal representation.

4. For each ARCNET node ID that is not present on the network, the system shall generate a message indicating the node ID and its hexadecimal representation.

## Application Execution Management

1. The system shall initialize the stack at startup.

2. The system shall perform initialization of the primary system components.

3. The system shall initialize the cyclic redundancy check module.

4. The system shall initialize the flash memory module.

5. The system shall initialize the bit module.

6. The system shall initialize the interrupt system.

7. The system shall initialize the I2C controller.

8. The system shall initialize the weapon system.

9. The system shall initialize the weapon settings.

10. The system shall load weapon data into the auxiliary string buffer.

11. The system shall initialize the MCP module using the auxiliary string buffer.

12. The system shall initialize the auxiliary module using the auxiliary string buffer.

13. The system shall initialize the IF module using the auxiliary string buffer if the fuze card is fitted.

14. The system shall initialize the MLAN module using the auxiliary string buffer.

15. The system shall initialize the telemetry module.

16. The system shall initialize the test equipment.

17. The system shall initialize the mission using the auxiliary string buffer.

18. The system shall initialize the fuze module using the auxiliary string buffer if the fuze card is fitted.

19. The system shall initialize the servos using the auxiliary string buffer.

20. The system shall initialize the WiFi module using the auxiliary string buffer.

21. The system shall initialize the battery pack using the auxiliary string buffer.

22. The system shall initialize the HOBS module using the auxiliary string buffer if the HOBS is present.

23. The system shall initialize the guidance module.

24. The system shall initialize the navigation module.

25. The system shall initialize the task manager.

26. The system shall clear the CBIT.

27. The system shall enable interrupts.

28. The system shall output a message indicating GCA initialization completion using the auxiliary string buffer and the current time in microseconds.

29. The system shall service the auxiliary transmission using the HILS ADS receive UART.

30. The system shall check communications with the IMU.

31. The system shall execute the guidance at 10 Hertz.

32. The system shall send aircraft data if the DTAC is present and the DBAS is enabled.

33. The system shall send WIAC82 data if the DTAC is present and the WIAC82 is enabled.

34. The system shall output the WIAC82 string buffer if the WIAC82 string buffer has content.