# Capability Requirements

## EGM96 Module Capability Requirements

EGM96 Height Calculation.

The P3 MCP EGM96 Module shall provide the following capabilities:

EGM96 Geoid Height Calculation Module

## EGM96 Geoid Height Calculation Module

1. The EGM96 height calculation module shall compute a geoid height value based on input latitude and longitude coordinates.

2. The EGM96 height calculation module shall convert the input latitude value from radians to degrees and add 90.0 before multiplying by 2.

3. The EGM96 height calculation module shall convert the input longitude value from radians to degrees before multiplying by 2.

4. The EGM96 height calculation module shall determine a latitude grid index by taking the integer portion of the adjusted latitude value.

5. The EGM96 height calculation module shall determine a longitude grid index by taking the integer portion of the adjusted longitude value.

6. The EGM96 height calculation module shall validate that the calculated latitude grid index is less than 360 and the longitude grid index is less than 720.

7. The EGM96 height calculation module shall calculate a latitude offset factor by subtracting the latitude grid index from the adjusted latitude value.

8. The EGM96 height calculation module shall calculate a longitude offset factor by subtracting the longitude grid index from the adjusted longitude value.

9. The EGM96 height calculation module shall perform linear interpolation between adjacent longitude grid points for a given latitude grid index and store the result in a first intermediate value.

10. The EGM96 height calculation module shall perform linear interpolation between adjacent longitude grid points for the next latitude grid index and store the result in a second intermediate value.

11. The EGM96 height calculation module shall perform a second interpolation between the two intermediate values using the latitude offset factor to determine the final geoid height.

12. The EGM96 height calculation module shall return a success indicator of 1 when the input coordinates are within the valid range.

13. The EGM96 height calculation module shall set the output geoid height value to 0.0 when the input coordinates are outside the valid range.

14. The EGM96 height calculation module shall return a failure indicator of 0 when the input coordinates are outside the valid range.