# Capability Requirements

## I2C_Controller Module Capability Requirements

I2C Controller module.

The P3 MCP I2C_Controller Module shall provide the following capabilities:

I2C Controller Initialization State

I2C Device Polling Configuration

I2C Controller Polling Operation

Received Data Retrieval Mechanism

Elapsed Time Indicator Reading Capability

ETI Device Reset Capability

## I2C Controller Initialization State

1. The I2C controller module shall initialize the internal controller structure by clearing all bytes of the memory space allocated for the controller.

2. The I2C controller module shall initialize the list of devices to be polled by clearing all bytes of the memory space allocated for the list.

3. The I2C controller module shall set the index used for tracking the current polling device to zero.

4. The I2C controller module shall set the count of devices to be polled to zero.

5. The I2C controller module shall set the polling cycle state to the write state.

6. The I2C controller module shall initialize the first I2C controller instance with the base address 0x45400000 + 0x08000.

7. The I2C controller module shall initialize the second I2C controller instance with the base address 0x45400000 + 0x10000.

8. The I2C controller module shall initialize the third I2C controller instance with the base address 0x45400000 + 0x18000.

9. The I2C controller module shall configure the ETI device to use the first I2C controller instance and assign the device identifier 0x6B.

10. The I2C controller module shall configure the 5V power supply device to use the first I2C controller instance and assign the device identifier 0x40.

11. The I2C controller module shall configure the 1V power supply device to use the first I2C controller instance and assign the device identifier 0x42.

12. The I2C controller module shall configure the 1.35V power supply device to use the first I2C controller instance and assign the device identifier 0x48.

13. The I2C controller module shall configure the 1.8V power supply device to use the first I2C controller instance and assign the device identifier 0x43.

14. The I2C controller module shall configure the 3.3V power supply device to use the first I2C controller instance and assign the device identifier 0x4C.

15. The I2C controller module shall configure the 28V power supply device to use the second I2C controller instance and assign the device identifier 0x41.

16. The I2C controller module shall configure the 6V GCA power supply device to use the second I2C controller instance and assign the device identifier 0x46.

17. The I2C controller module shall configure the 5V IMU power supply device to use the second I2C controller instance and assign the device identifier 0x44.

18. The I2C controller module shall configure the 5V EFUZE power supply device to use the second I2C controller instance and assign the device identifier 0x45.

19. The I2C controller module shall configure the first pressure sensor's pressure measurement device to use the second I2C controller instance and assign the device identifier 0x76.

20. The I2C controller module shall configure the first pressure sensor's temperature measurement device to use the second I2C controller instance and assign the device identifier 0x76.

21. The I2C controller module shall configure the second pressure sensor's pressure measurement device to use the second I2C controller instance and assign the device identifier 0x77.

22. The I2C controller module shall configure the second pressure sensor's temperature measurement device to use the second I2C controller instance and assign the device identifier 0x77.

23. The I2C controller module shall add the first pressure sensor's pressure measurement device to the polling list with a command byte of 0x46, a read length of one byte, and no delay.

24. The I2C controller module shall add the second pressure sensor's pressure measurement device to the polling list with a command byte of 0x46, a read length of one byte, and no delay.

25. The I2C controller module shall add the 5V power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

26. The I2C controller module shall add the 1V power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

27. The I2C controller module shall add the 1.35V power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

28. The I2C controller module shall add the 1.8V power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

29. The I2C controller module shall add the first pressure sensor's pressure measurement device to the polling list with no command byte, a read length of zero bytes, and a delay of three milliseconds.

30. The I2C controller module shall add the second pressure sensor's pressure measurement device to the polling list with no command byte, a read length of zero bytes, and a delay of three milliseconds.

31. The I2C controller module shall add the first pressure sensor's temperature measurement device to the polling list with a command byte of 0x56, a read length of one byte, and no delay.

32. The I2C controller module shall add the second pressure sensor's temperature measurement device to the polling list with a command byte of 0x56, a read length of one byte, and no delay.

33. The I2C controller module shall add the 3.3V power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

34. The I2C controller module shall add the 28V power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

35. The I2C controller module shall add the 6V GCA power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

36. The I2C controller module shall add the 5V IMU power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

37. The I2C controller module shall add the 5V EFUZE power supply device to the polling list with no command byte, a read length of zero bytes, and a write length of two bytes.

38. The I2C controller module shall add the first pressure sensor's temperature measurement device to the polling list with no command byte, a read length of zero bytes, and a delay of three milliseconds.

39. The I2C controller module shall add the second pressure sensor's temperature measurement device to the polling list with no command byte, a read length of zero bytes, and a delay of three milliseconds.

40. The I2C controller module shall initialize the first pressure sensor with the second I2C controller instance and assign the device identifier 0x76.

41. The I2C controller module shall initialize the second pressure sensor with the second I2C controller instance and assign the device identifier 0x77.

## I2C Device Polling Configuration

1. The I2C controller shall allow adding up to 20 I2C devices to the poll list.

2. The I2C controller shall store the reference to the specified I2C device in the poll list.

3. The I2C controller shall associate the sub-address value with the added device in the poll list.

4. The I2C controller shall record the transmit byte count for the added device in the poll list.

5. The I2C controller shall record the receive byte count for the added device in the poll list.

6. The I2C controller shall copy the transmit buffer data into the poll list for the added device.

7. The I2C controller shall assign the poll index value to the specified I2C device based on the current poll list count.

## I2C Controller Polling Operation

1. The I2C controller shall determine whether to perform a write operation or a read operation based on the current polling cycle state.

2. The I2C controller shall initiate a write transaction to an I2C device if the transmit buffer contains data and the current polling cycle state is set to write.

3. The I2C controller shall transition to the read state if the transmit buffer is empty and the current polling cycle state is set to write.

4. The I2C controller shall check the NACK status of the I2C bus before initiating a read operation.

5. The I2C controller shall initiate a read transaction to an I2C device if the NACK status is not asserted and the receive buffer contains data.

6. The I2C controller shall transition to the get data state and return a success indicator if the receive buffer is empty and the NACK status is not asserted.

7. The I2C controller shall return a failure indicator if the NACK status is asserted during a read operation.

8. The I2C controller shall retrieve received data from the I2C bus if the current polling cycle state is set to get data and the receive buffer contains data.

9. The I2C controller shall reset the polling cycle state to the write state if the polling cycle exceeds the get data state.

10. The I2C controller shall increment the polling cycle state if a successful operation occurs during the polling cycle.

11. The I2C controller shall reset the failure flag and failure counter if a successful operation occurs during the polling cycle.

12. The I2C controller shall set the failure flag and increment the failure counter if an operation fails during the polling cycle.

13. The I2C controller shall transition to the get data state plus one if an operation fails during the polling cycle.

14. The I2C controller shall skip the current device if the failure counter exceeds a threshold of 3 consecutive failures.

15. The I2C controller shall increment the polling index and reset the polling cycle state to the write state after completing the service of a device.

16. The I2C controller shall reset the polling index to zero and return a success indicator if all devices have been serviced during a polling cycle.

## Received Data Retrieval Mechanism

1. The I2C controller shall return zero if the poll index of the specified device is greater than or equal to the total number of devices to poll.

2. The I2C controller shall retrieve the poll information structure corresponding to the poll index of the specified device.

3. The I2C controller shall return zero if the failure flag in the poll information structure is set.

4. The I2C controller shall copy the received data buffer contents into the provided receive data buffer.

5. The I2C controller shall return the count of received bytes from the poll information structure.

## Elapsed Time Indicator Reading Capability

1. The I2C Controller module shall read data from the ETI device using the sub-address 5.

2. The I2C Controller module shall perform a read operation on the ETI device to retrieve 4 bytes of data.

3. The I2C Controller module shall combine the 4 received bytes into a 32-bit value using bit shifting and addition.

4. The I2C Controller module shall right-shift the combined 32-bit value by 2 bits.

5. The I2C Controller module shall return the resulting value as the elapsed time indicator.

6. The I2C Controller module shall return the value 0xFFFFFFFF if the read operation fails.

## ETI Device Reset Capability

1. The I2C controller shall transmit two bytes of data to the ETI device during the reset operation.

2. The first byte of data transmitted shall have a value of 0x00.

3. The second byte of data transmitted shall have a value of 0x04.

4. The I2C controller shall use the I2C interface identified by the index associated with the ETI device for communication.

5. The I2C controller shall target the device identified by the address stored for the ETI device during the reset operation.

6. The I2C controller shall perform a write operation without sending a start condition before the data transmission.

7. The I2C controller shall indicate that the operation is a repeated start condition.

8. The I2C controller shall configure the data transfer to be a write operation.