# Capability Requirements

## NVRAM Module Capability Requirements

NVRAM module is responsible for managing the access to non-volatile memory used for storing critical system parameters,
configuration data, logs, and persistent state across power cycles.

The P3 MCP NVRAM Module shall provide the following capabilities:

NVRAM Data Storage Management

NVRAM Data Retrieval Capability

## NVRAM Data Storage Management

1. The NVRAM module shall reject write requests if the sum of the address and length exceeds the maximum address limit of 0x7FFF.

2. The NVRAM module shall format the provided address into a two-byte sub-data array where the most significant byte is derived from the upper 8 bits of the address and the least significant byte is derived from the lower 8 bits of the address.

3. The NVRAM module shall initiate an I2C write operation using the I2C channel 2 with the device identifier 0x50, transmitting the formatted address bytes followed by the data bytes, and shall return a success status if the operation completes successfully.

4. The NVRAM module shall return a failure status if the I2C write operation does not complete successfully.

## NVRAM Data Retrieval Capability

1. The NVRAM module shall reject any read request that attempts to access an address beyond the maximum allowed address of 0x7FFF.

2. The NVRAM module shall perform an I2C read operation on the I2C bus 2 using the device ID 0x50 to retrieve data from the specified address.

3. The NVRAM module shall return a value of 1 when the read operation is successfully completed.

4. The NVRAM module shall return a value of 0 when the read operation fails or encounters an error.